Name: ______________________________________	Date: ________________________

Student Exploration: Wheel and Axle

Vocabulary: effort, force, load, mechanical advantage, radius, wheel and axle

Prior Knowledge Questions (Do these BEFORE using the Gizmo.)

- A ship captain uses a steering wheel (called a “helm”) to turn the rudder of a sailing ship. Do you think it would be easier for him to turn a large wheel or a small wheel? Explain why you think so.

___________________________________________________

___________________________________________________

- Which screwdriver do you think would turn a screw most easily? Circle your choice and explain why you picked the one you did.

_________________________________________________________________________

_________________________________________________________________________

Gizmo Warm-up: Dragging rocks

The Wheel and Axle Gizmo™ shows a simple machine called a wheel and axle. In this machine, turning the outer wheel will also turn the inner axle. The axle is connected to a rope that pulls a load of rocks.

- Drag a barbarian to the wheel and press Play (). The barbarian pushes on the wheel with a certain force. Is the barbarian strong enough to move the wheel?

_____________________________________________

- Click Reset (). Drag more barbarians to the wheel. The total force that the barbarians exert on the wheel is called the effort.

How many barbarians does it take to move the load of rocks? ________________________

| Activity A: Wheel radius | Get the Gizmo ready: - Click Reset. - Remove all barbarians from the wheel. |  |
| --- | --- | --- |

Question: How does the Wheel radius affect the force needed to turn the wheel?

- Observe: The radius of the wheel is the distance from the center of the wheel to the edge. Use the Gizmo to explore the question given above. (You can tell how much force is needed to turn the wheel by checking the smallest number of barbarians it takes to move it.)

- Collect data: Find the smallest number of barbarians needed to turn the wheel for each combination listed below. Click Reset between each trial.

| Wheel radius | Axle radius | Smallest number of barbarians needed (force) | Distance traveled by barbarians (m) | Time needed to move the load (sec) |
| --- | --- | --- | --- | --- |
| 6 m | 4 m |  |  |  |
| 8 m | 4 m |  |  |  |
| 10 m | 4 m |  |  |  |
| 12 m | 4 m |  |  |  |

- Analyze: How did the effort needed to turn the wheel change as the wheel radius increased?

_________________________________________________________________________

_________________________________________________________________________

- Extend your thinking: The wheel and axle is helpful because it reduces the effort needed to move a load. What is the “price” you have to pay for this help?

_________________________________________________________________________

_________________________________________________________________________

- Work:  Work is the force * distance traveled.

  - How much work was done by the barbarians when the wheel radius was 6m? ________

  - How much work was done by the barbarians when the wheel radius was 10m? ________

- Power:  Power is the Work / time

- What was the power when the wheel radius when the wheel radius was 6m? ________

| Activity B: Mechanical advantage | Get the Gizmo ready: - Click Reset. - Set the Wheel radius to 10 meters. - Set the Axle radius to 5 meters. |  |
| --- | --- | --- |

Question: How does a wheel and axle multiply force?

- Collect data: For each case, use the Gizmo to determine the smallest number of barbarians needed to turn the wheel and the distance they walked. Click Reset between each trial.

| Wheel radius | Axle radius | Smallest number of barbarians needed | Distance barbarians walked (m) | Time needed (sec) |
| --- | --- | --- | --- | --- |
| 10 m | 5 m |  |  |  |
| 14 m | 7 m |  |  |  |
| 6 m | 3 m |  |  |  |

- Analyze: What do you notice? _________________________________________________

_________________________________________________________________________

- Work:

- How much work was done by the barbarians when the wheel radius was 10m? ________

- How much work was done by the barbarians when the wheel radius was 14m? ________

- Power:

- What was the power when the wheel radius when the wheel radius was 14m? ________

- Calculate: The mechanical advantage (MA) is a number indicating how much a simple machine reduces the effort needed to move a load. The mechanical advantage of a wheel and axle is roughly equal to the Wheel radius divided by the Axle radius.

- What is the MA when the wheel radius is 8 m and the axle radius is 2 m? _______________

- Collect data: For each setup below, calculate the mechanical advantage and record the smallest number of barbarians it takes to move the wheel.

| Wheel radius | Axle radius | Mech. advantage | Smallest # of barbarians needed |
| --- | --- | --- | --- |
| 12 m | 3 m |  |  |
| 6 m | 1 m |  |  |
| 15 m | 5 m |  |  |

- Analyze: In each example, multiply the mechanical advantage by the number of barbarians.

- What pattern do you notice? _______________________________________________

- Challenge: How many barbarians would it take to move the load if there were no wheel and axle? (Hint: MA = 1.) __________________________________________________

DON’T FORGET TO DO THE ASSESSMENT QUESTIONS!!!

(and Reflections if you miss any of the questions!!)